NOTES and Questions are in GREEN

Mantis item 2037

32.4.3	Setting parameters in configurations

QUESTION: Should this proposal be new section 32.4.3 or new section 32.9 – is there a preference?  Does it matter?

Configurations can be used to override parameters declared within a design, or to apply or override parameters values for specific instantiations of a design.  When a common value is desired for overriding a number of parameters in a configuration, a parameter or a localparam can be declared in the configuration.

The following is a list of detailed restrictions regarding setting parameters from a configuration:

-- A parameter or a localparam declared in a configuration must be assigned a value and shall only be set to a literal value.

-- Index expressions in a hierarchical name shall only refer to literals or localparams of the configuration.

-- If a reference to an identifier in the target instance context is used in an override, it shall be the only term in the expression.

-- A hierarchical identifier in a parameter override shall be resolved starting in the context of the configured instance.  If a hierarchical identifier is used, it must be the only term in the expression, i.e., a.b.c + 7 is invalid.

-- A parameter override in a configuration shall not refer to a constant function other than a predefined constant system function.

-- A parameter override from a configuration shall take precedence over a defparam statement when both are referencing the same parameter at the same level of hierarchy.  In all other conditions, defparam statements work as defined.

-- Configurations may not use positional parameter notation to override parameters.

The two following two code sample modules are used by examples in this subclause.  Note that module top includes an instantiation of adder and does not include any direct parameter modifications.

module adder #(parameter 	ID  = “id”,

W     = 8,

D     = 512)(...);

...

$display(“ID = %s,  W = %d,  D = %d”, ID, W, D);

...

endmodule:adder

Example 1

module top (...);

parameter WIDTH = 16;

adder a1 (...);

endmodule

Example 2

The code above, without any parameter overrides on adder a1, will print:

ID = id, W=8, D = 512

A configuration can be used to override a parameter in a module and apply that overridden parameter to instantiations within that module.  The configuration in Example 3 overrides the default value of parameter WIDTH in module top shown in Example 2.  Example 3 then uses parameter WIDTH to override the parameter for the instantiation of adder.  The code for adder is found in Example 1.

config cfgl;

design rtlLib.top;

instance top use #(.WITDH(32));

instance top.a1 use #(.W(top.WIDTH));

endconfig

Example 3

Using configuration cfg1 above, the $display in adder will print:

ID = id, W=32, D = 512

A parameter or localparam can be used in a configuration to represent a value sent to multiple instances.

module top4 ();

parameter WIDTH = 16;

adder a1 #(.ID(“a1”))(...);

adder a2 #(.ID(“a2”))(...);

adder a3 #(.ID(“a3”))(...);

adder a4 #(.ID(“a4”))(...);

endmodule

config cfg2;

localparam S = 24

design rtlLib.top4;

instance top4.a1 use #(.W(S));

instance top4.a2 use #(.W(S));

endconfig

Example 4

With cfg2 configuring module top4.a1, the $display in adder will print:

ID = a1, W=24, D = 512

ID = a2, W=24, D = 512

ID = a3, W=8, D = 512

ID = a4, W=8, D = 512

Note that in this example, the override of adder port ID in top4 is maintained.

A parameter override with empty parentheses will set the parameter back to its default as defined in its module.

module top5 (...);

parameter WIDTH = 64, DEPTH = 1024, ID = “FOO”;

adder a1 #(.ID(ID), .W(WIDTH), .D(DEPTH))(...);

endmodule

Example 5

Using the code above, the $display in adder will print:

ID = FOO, W=64, D = 1024

config cfg3;

design rtlLib.top5;

instance top5.a1 use #(.W()); // set only parameter W

// back to it’s default

endconfig

Example 6

When the above configuration cfg3 configures module top5.a1, the $display in adder will print:

ID = FOO, W=8, D = 1024

Only parameter W is configured to use it’s default value.

All the parameters can be reset to their default values, meaning their initial values, prior to any parameter overrides.  This is shown in the following configuration, which uses the module top from Example 2.

config cfg4;

design rtlLib.top;

instance top.a1 use #();  // set all parameters in

//instance a1 back to their defaults

endconfig

Example 7

This configuration of  top.a1 will print:

ID = id, W=8, D = 512

When both a configuration and a defparam statement are modifying the same parameter at the same hierarchal level, the configuration will take precedence.  The following code adds a level of hierarchy which includes defparam statements.

module test;

...

top8 t(...);

defparam t.WIDTH = 64;

defparam t.a1.W = 16;

...

endmodule;

module top8 (...);

parameter WIDTH = 32;

adder a1 #(.ID(“a1”)) (...);

adder a2 #(.ID(“a2”),.W(WIDTH))(...);

endmodule

module adder #(parameter 	ID  = “id”,

W     = 8,

D     = 512)(...);

...

$display(“ID = %s,  W = %d,  D = %d”, ID, W, D);

...

endmodule

config cfg6;

design rtlLib.test;

instance test.t    use #(.WITDH(48));

endconfig

Example 8

With cfg6 configuring module top8.a1, the $display in adder will print:

ID = a1, W=16, D = 512

ID = a2, W=48, D = 512