IEEE P1364.1 Verilog Synthesis Interoperability Working Group

Conference Call Minutes

August 10, 2001

Minutes taken by Ken Coffman

Send comments and corrections to kcoffman@sos.net

Callers:

J. Bhasker

Cliff Cummings

Stefen Boyd

Ben Cohen

Ken Coffman

Sashi Obilisetty

· Should we type checking, like mismatched registers? Lint tools report this type of problem. Not a synthesis issue, more of a design issue. Some folks hate VHDL types and casting. Consensus seems to be leave Verilog with loose typing.

· Named parameter association is a nice format feature, shouldn’t be a problem for tool vendors. An example needs to be created to illustrated the usage (Assigned to Ken Coffman).

· Conditional compilation is clearer in the VHDL LRM. For 1364.1 we need a good example with good indentation. No issues associated with this feature. `ifndef, `elsif, `undef

· `line compiler directive needs an example, PLI implementation, no synthesis issue (ignored?).

· @* event declaration fully supported, can be used inside always block. Allow use of ‘,’ instead of ‘or’?

· The 1364 group is hoping that defparams will die, named parameter associations are permitted. The problem is that defparams can be encountered anywhere. Any child module can affect modules higher in the hierarchy. The same parameter might be used in multiple modules. Named association works well. Indirect effects of defparam hard to trace and report. 1364.1 should include wording which discourages buried defparams. 

· Ben Cohen is working on RAM/ROM examples. There are options for implementing memory (Block RAM. Block ROM, Registers/latches) which should be selected via attributes. Suggest use of funny brace attributes (* attrib here *). Other options include async and sync RAM, single and dual port. Ben and Cliff will hash this out.

· A method of initializing RAM/ROM would be desirable. Use initial block or $Readmem? Unfortunately, $Readmem is not used by synthesis tools. Use of initial is a can of worms. Initial assignments can be globally static?

· Define a real Verilog constant? Local param = constant can’t be an array. In dueling case statements, the last assignment wins. Can we have 3 case statements? Check in with Synplicity and Synopsys contacts.

· The next conference call will be on Friday, Sept 7.

